Homework 2:

Network Design and Configuration

In this assignment, I will outline the design and configuration of a network for the company I am working with as a technician. My primary focus is on ensuring traffic security and availability for every device within the network. Additionally, we anticipate that each department will have between five to ten new employees, necessitating careful planning of the IP addresses to accommodate them.

Department Specifications

The network will be structured according to the following departmental specifications:

- Human Resources (HR) Department: This department will have one office equipped with 10 PCs.

- Information Technology (IT) Department: This department will consist of two separate offices, totaling 15 devices, along with one printer.

- Administration Department: Comprising three offices, this department will host a total of 20 PCs and two printers.

- Server Room: This area includes two offices containing one PC, as well as a DNS server, a web server, and an FTP server.

To facilitate IP configuration management for all devices within the network, I will incorporate a DHCP server. This will streamline our processes as technicians and simplify IP management.

Network Design

For a robust network design, I propose implementing a star topology, featuring a Layer 3 core switch that connects to each department. Within the Administration Department, which consists of three rooms, we will have a total of 15 devices, including the printers. Each device in a room will connect to a dedicated switch located within that room.

The same structure will apply to the IT Department, where two rooms will each have a switch connected to the devices. The HR Department will also follow this model, with its own switch and devices. In the server room, we will utilize two rooms, each equipped with a Layer 2 switch connected to the servers and PCs.

To enhance security and availability across the network, I plan to segment the company’s network into subnetworks, which will improve communication and collaboration among devices within each department. To achieve this, I will configure Virtual LANs (VLANs) for each department:

- Administration Department: VLAN 10

- IT Department: VLAN 20

- HR Department: VLAN 30

- Server Room: VLAN 40

Configuration Steps

- Creating VLANs on the Core Switch:

  - Access the core switch and enter configuration mode.

  - For VLAN 10 (Administration), use the following commands:

vlan 10

name Admin

exit

  - Repeat the process for VLAN 20 (IT), VLAN 30 (HR), and VLAN 40 (Server Room) similarly.

- Assigning VLANs to Each Switch:

  - Go to each switch corresponding to the rooms and assign the appropriate VLAN:

    - For example, for Switch 1 (Room 1):

en

conf t

vlan 10

name Admin

exit

    - Repeat for Switches in Rooms 2 and 3 for the Administration Department and continue the same process for the IT and HR departments.

- Configuring Access Ports:

  - For each switch, configure the interfaces connected to devices:

En

Conf t

interface range fa0/1 - 10 (adjust based on the number of devices that’s connected to devices)

switchport mode access

switchport access vlan 10

exit

- Setting Up Trunk Ports:

  - Configure trunk ports on switches that connect to the core switch:

En

Conf t

interface fa0/1 (specify the interface connected to the core switch)

switchport mode trunk

exit

  - On the core switch, configure the interfaces connecting to each room switch:

en

conf t

interface range fa0/1 - 8

switchport mode trunk

exit

- Enabling Inter-VLAN Routing:

  - On the core switch, enable IP routing to facilitate communication between VLANs:

En

Conf t

ip routing

- Assigning IP Addresses to VLAN Interfaces:

  - Assign IP addresses to each VLAN on the core switch:

En

Conf t

interface vlan 10

ip address 192.168.10.1 255.255.255.0

no shutdown

exit

  - Repeat for VLANs 20, 30, and 40 with appropriate IP addresses.

  - Vlan20(192.168.20.1), Vlan30(192.168.30.1), Vlan40(192.168.40.1).

- Configuring the DHCP Server:

  - Set a static IP address for the DHCP server within VLAN 40:

En

Conf t

IP address: 192.168.40.2

Subnet Mask: 255.255.255.0

Default Gateway: 192.168.40.1

  - Create DHCP pools for each department, ensuring that each pool has a unique range of IP addresses and the corresponding default gateway.

Pool Name: Admin

Default Gateway: 192.168.10.1

Starting IP Address: 192.168.10.2

Subnet Mask: 255.255.255.0

DNS Server: 192.168.40.2 (or another DNS server)

Same appley to IT, HR, SERVER dep.

- Configuring DHCP Relay:

  - On the core switch, use the ip helper-address command to point to the DHCP server for each VLAN:

En

Conf t

interface vlan 10

ip helper-address 192.168.40.2

exit

  - Repeat this for VLANs 20 and 30.

Testing Connectivity

Once all configurations are complete, I will test connectivity by randomly changing the IP configuration on devices in the IT, HR, and Administration departments to dynamic. I will verify successful IP address allocation through DHCP and ensure devices can communicate across VLANs by performing ping tests.

The implementation of VLANs enhances network efficiency by segmenting traffic and improving security. Devices in VLAN 10, for example, cannot communicate with devices in VLAN 20 without proper permissions, thus reinforcing the network's integrity.

Implementing Routing Protocols

In this section, we aim to identify the most suitable routing protocol for our network. Given that we are working with a small company that consists of three departments and has a maximum of 40 to 45 devices, this setup qualifies as a small network. Our routing protocol choices will depend on the specific needs of the company.

For our requirements, both Static Routing and the Routing Information Protocol (RIP) are viable solutions. While static routing must be configured manually, which can be time-consuming as the network grows, RIP offers a more dynamic solution. As a protocol designed for small to medium-sized networks, RIP is straightforward to configure and automatically updates routing tables every 30 seconds. It operates using the distance vector routing method, which relies on hop count to determine the best path between source and destination devices. The algorithm identifies directly connected neighbors to calculate the optimal route based on the fewest hops.

To configure the RIP protocol on our core switch, enter the configuration mode and use the following commands:

- Enable RIP:

router rip

- Specify the RIP Version:

version 2

This command selects RIP version 2, which has some advantages over version 1.

- Add Networks:

network 192.168.10.0

network 192.168.20.0

network 192.168.30.0

network 192.168.40.0

This will add the networks associated with VLANs 10, 20, 30, and 40 to the RIP protocol.

- Disable Auto-Summarization:

no auto-summary

This command prevents RIP from summarizing the subnets into a single large network, ensuring accurate routing for each VLAN.

Implementing Data Traffic Rules

Implementing data traffic rules in our network is crucial for optimizing performance and ensuring security. Here are some key rules to consider:

1. Segmentation Using VLANs:
VLANs help isolate network traffic, reducing broadcast traffic and enhancing security within the organization. To configure this on the core switch, enter configuration mode and use the following command:

access-list 100 permit ip 192.168.10.0 0.0.0.255 192.168.30.0 0.0.0.255

This command allows the administration network to access the server network. To deny access from other networks, use:

access-list 100 deny ip any any

interface VLAN 40

ip access-group 100 in

This configuration ensures that all other networks cannot access the server network.

- Access Control and Security Rules:
Like the segmentation VLANs, access control rules can restrict connectivity between specific networks. By implementing similar access lists as in the first step, we can ensure that only authorized networks can access sensitive areas of the network.

- Firewall Rules and Security:
The objective here is to secure our network while controlling traffic flow. For internet access management, use the following command:

ip access-list extended block_server_internet

deny ip any 192.168.40.0 0.0.0.255

permit ip any any

interface VLAN 40

ip access-group block_server_internet in

This command blocks access from any network to the server network while allowing other traffic. Block server internet is a list used to add all the network been not accessible from internet.

To ensure effective configuration and maintenance within the organization, it is essential for the IT department to have access to all VLANs. Therefore, we must grant the IT department the necessary permissions.

To achieve this, we can implement the following access control list (ACL) commands:

access-list 100 permit ip 192.168.20.0 0.0.0.255 192.168.10.0 0.0.0.255

access-list 100 permit ip 192.168.20.0 0.0.0.255 192.168.30.0 0.0.0.255

access-list 100 permit ip 192.168.20.0 0.0.0.255 192.168.40.0 0.0.0.255

access-list 100 deny ip any any

These commands will allow the IT department (VLAN 20) to access the admin (VLAN 10), HR (VLAN 30), and Server (VLAN 40) VLANs while denying access to any other networks.

Next, we need to apply this access list to the appropriate VLAN interfaces:

interface VLAN 10

ip access-group 100 in

exit

interface VLAN 30

ip access-group 100 in

exit

interface VLAN 40

ip access-group 100 in

exit

By implementing these rules, we enhance the security of our network while ensuring that the IT department can efficiently perform their tasks across all VLANs.