# Packet Tracer - Create a LAN

# Addressing Table

| Device | Interface/Port | IPv4 Address | Subnet Mask |
| --- | --- | --- | --- |
| Admin PC | NIC | DHCP | N/A |
| Manager PC | NIC | DHCP | N/A |
| Printer | NIC | 192.168.1.100 | 255.255.255.0 |
| www.cisco.pt | NIC | 209.165.200.225 | N/A |

Blank Line, No additional information

# Objectives

=   Connect Network Devices and Hosts

=   Configure Devices with IPv4 Addressing

=   Verify the End Device Configuration and Connectivity

=   Use Networking Commands to View Host Information

# Background / Scenario

A new branch office is opening, and you have been asked to set up the LAN. The network devices are already set up, but you need to connect them and the hosts together. You also need to configure IPv4 addressing on the end devices and verify that they can reach local and remote resources.

# Instructions

## Part 1: Connect Network Devices and Hosts

### Step 1: Power on the end devices and Office Router.

a.     Click each device and open its Physical Tab. Note: There is no power switch on the switch model used in this activity.

b.     Locate the power switch for each device in the Physical Device View window.

c.     Click the power switch to turn the device on. You should see a green light near the power switch to indicate that the device is powered on.

### Step 2: Connect the end devices.

Use the table and instructions to connect the network devices and hosts to create the physical network.

Connections Table

| Device | Interface/Port | Connected to Device | Connection Interface/Port |
| --- | --- | --- | --- |
| Office Router | G0/0 | ISP1 | G0/0 |
| Office Router | G0/1 | Switch | G0/1 |
| Admin PC | NIC (F/0) | Switch | F0/1 |
| Manager PC | NIC (F/0) | Switch | F0/2 |
| Printer | NIC (F/0) | Switch | F0/24 |

Blank Line, No additional information

Note: In the table above, interfaces designated with G are GigabitEthernet interfaces. Interfaces that are designated with F are FastEthernet interfaces.

a.     Connect the networking devices according to the information in the Connections Table using Ethernet copper straight-through cables. For the internet to Office Router connection, select the device and port from the dropdown menus that appear when you click the cloud with connections tool selected.

b.     Connect the two PCs and the printer to the office switch according to the information in the connections table. Use copper straight-through cables.

c.     You should see green link lights on all connections after a brief delay.

## Part 2: Configure Devices with IPv4 Addressing

### Step 1: Configure the hosts with addressing information.

a.     The Admin and Manager PCs should receive their IP addressing information from DHCP. The Office Router has been configured to supply IP addresses to hosts on the branch office LAN. Click the PCs and go to the Desktop tabs on each PC. Open the IP Configuration application and configure the PCs to receive their IP addresses dynamically.

b.     Printers and servers are often manually configured with addressing because other devices on the network are configured to access them using IP addresses. Manual configuration with a static address will ensure that the IP addresses of these devices do not change.

1)    Click the printer and open the Config tab.

2)    Click the FastEthernet0 interface in the left-hand pane.

3)    Enter the addressing information from the Addressing Table.

c.     Because the two computers are on the same network, their IPv4 addresses will be similar, their subnet masks and default gateways will be identical.

#### Questions:

Why do you think the IPv4 addresses are different, but the subnet masks and default gateways are the same?

The IP address represent each identity the network therefore it should be deferent and unique!

The subnet masks refer to the grouping in the same network and how many devices in this network and its capacity. All devices located in the same network so subnet masks should be same! Same thing applies to default gateways due to all devises located In the same network and will have the same gateways to the router.

The printer does not require a default gateway because it will only be accessed by hosts on the local network. However, if you need to configure it with a default gateway, which value will the printer use? How can you determine this from the other devices in the network?

Due that the printer located in the same network with same subnet, it will use the same gateway to access the internet. This applied for all devices in the same network.

## Part 3: Verify the End Device Configuration and Connectivity

### Step 1: Verify connectivity between the two PCs.

a.     Go to the desktops of the PCs and check the IP addressing configuration. You should see that the PCs have dynamically received IP addresses on the 192.168.1.0 255.255.255.0 network. You should also see that they have received addresses for the Default Gateway and DNS server settings.

b.     From the command prompt on Admin PC, ping the IP address of the Printer. Repeat this process for the Manager PC. You should see successful pings for each. This verifies that the PCs and the printer are powered on, and correctly connected and addressed.

### Step 2: Verify connectivity to the internet.

From the desktop of the PCs, open the Web Browser. Enter the IP address of the internet server to display the webpage. Repeat the process but connect using the URL of the server.

#### Question:

If you can connect by the IP address, but not by the URL, what do you think is the cause of this problem?

For me, when I use the IP address, its show the WEB PAGE WITH ITS CONTAIN. But using the URL the page in the first attempt didn’t show anything and reason behind that is DNS server didn’t set yet. At second attempt its show the page info. The question is why its shows the info in the second attempt? Note that I’m using mac.

## Part 4: Use Networking Commands to View Host Information

The networking commands that are available from command prompt on PCs are very similar to those that are available in Windows. In this part of the activity, you will use ipconfig and traceroute to learn more about the LAN.

### Step 1: Use the ipconfig command.

The ipconfig command displays details about the addressing that is configured on a host.

#### Question:

Open a command prompt on one of the PCs and enter the ipconfig command and make note of the information that is returned. Now enter the ipconfig /all command. What additional information is displayed?

IPCONFIG/ALL

FastEthernet0 Connection:(default port)

Connection-specific DNS Suffix..:

Physical Address................: 0001.43BD.D368

Link-local IPv6 Address.........: FE80::201:43FF:FEBD:D368

IPv6 Address....................: ::

IPv4 Address....................: 192.168.1.3

Subnet Mask.....................: 255.255.255.0

Default Gateway.................: ::

192.168.1.1

DHCP Servers....................: 192.168.1.1

DHCPv6 IAID.....................:

DHCPv6 Client DUID..............: 00-01-00-01-DC-6C-63-D7-00-01-43-BD-D3-68

DNS Servers.....................: ::

209.165.200.225

Bluetooth Connection:

Connection-specific DNS Suffix..:

Physical Address................: 0001.42DA.DC65

Link-local IPv6 Address.........: ::

IPv6 Address....................: ::

IPv4 Address....................: 0.0.0.0

Subnet Mask.....................: 0.0.0.0

Default Gateway.................: ::

0.0.0.0

DHCP Servers....................: 0.0.0.0

DHCPv6 IAID.....................:

DHCPv6 Client DUID..............: 00-01-00-01-DC-6C-63-D7-00-01-43-BD-D3-68

DNS Servers.....................: ::

209.165.200.225

IPCONFIG

FastEthernet0 Connection:(default port)

Connection-specific DNS Suffix..:

Link-local IPv6 Address.........: FE80::201:43FF:FEBD:D368

IPv6 Address....................: ::

IPv4 Address....................: 192.168.1.3

Subnet Mask.....................: 255.255.255.0

Default Gateway.................: ::

192.168.1.1

Bluetooth Connection:

Connection-specific DNS Suffix..:

Link-local IPv6 Address.........: ::

IPv6 Address....................: ::

IPv4 Address....................: 0.0.0.0

Subnet Mask.....................: 0.0.0.0

Default Gateway.................: ::

0.0.0.0

### the red text in IPCONFIG/ALL are the additional info and DNS server IP and DHCP IP which is same as default getaway.

### Step 2: Use the traceroute command.

The tracerout command uses ICMP to return information about the routers that are passed as packets go from the source PC to the destination.

Trace to a remote destination by going to one of the PCs and entering tracert followed by the URL of the web server.

#### Questions:

How many routers are passed on the way to the destination? How are those routers identified?

Where is the second router located?

tracert www.cisco.pt

Tracing route to 209.165.200.225 over a maximum of 30 hops:

1 0 ms 0 ms 1 ms 192.168.1.1

2 0 ms 1 ms 0 ms 209.165.200.233

3 0 ms 0 ms 0 ms 209.165.200.225

Trace complete.

It passes 3 routers. They are identified by IP addresses. The second router located in the cloud and it’s the getaway for the server.

# Reflection

Consider a small office that has a LAN similar to the one you have created here. What is the biggest facilities challenge in setting up the network in a new location?

The challenges are the complexity of connecting many pcs or printers and how to manage and maintain the network.