TCP and UDP Communications

# Objectives

Part 1: Generate Network Traffic in Simulation Mode

Part 2: Examine the Functionality of the TCP and UDP Protocols

# Background

This simulation activity is intended to provide a foundation for understanding TCP and UDP in detail. Packet Tracer simulation mode provides you the ability to view the state of different PDUs as they travel through the network.

Packet Tracer Simulation mode enables you to view each of the protocols and the associated PDUs. The steps outlined below lead you through the process of requesting network services using various applications that are available on a client PC. You will explore the functionality of the TCP and UDP protocols, multiplexing, and the function of port numbers in determining which local application requested the data or is sending the data.

# Instructions

## Part 1: Generate Network Traffic in Simulation Mode and View Multiplexing

### Step 1: Generate traffic to populate Address Resolution Protocol (ARP) tables.

Perform the following task to reduce the amount of network traffic viewed in the simulation.

a.     Click MultiServer and click the Desktop tab > Command Prompt.

b.     Enter the ping -n 1 192.168.1.255 command. You are pinging the broadcast address for the client LAN. The command option will send only one ping request rather than the usual four. This will take a few seconds as every device on the network responds to the ping request from MultiServer.

c.     Close the MultiServer window.

### Step 2: Generate web (HTTP) traffic.

a.     Switch to Simulation mode.

b.     Click HTTP Client and open the Web Browser from the desktop.

c.     In the URL field, enter 192.168.1.254 and click Go. Envelopes (PDUs) will appear in the topology window.

d.     Minimize, but do not close, the HTTP Client configuration window.

### Step 3: Generate FTP traffic.

a.     Click FTP Client and open the Command Prompt from the desktop

b.     Enter the ftp 192.168.1.254 command. PDUs will appear in the simulation window.

c.     Minimize, but do not close, the FTP Client configuration window.

### Step 4: Generate DNS traffic.

a.     Click DNS Client and open the Command Prompt.

b.     Enter the nslookup multiserver.pt.ptu command. A PDU will appear in the simulation window.

c.     Minimize, but do not close, the DNS Client configuration window.

### Step 5: Generate Email traffic.

a.     Click E-Mail Client and open the E Mail tool from the Desktop.

b.     Click Compose and enter the following information:

1)    To: user@multiserver.pt.ptu

2)    Subject: personalize the subject line

3)    E-Mail Body: personalize the Email

c.     Click Send.

d.     Minimize, but do not close, the E-Mail Client configuration window.

### Step 6: Verify that the traffic is generated and ready for simulation.

There should now be PDU entries in the simulation panel for each of the client computers.

### Step 7: Examine multiplexing as the traffic crosses the network.

You will now use the Capture/Forward button in the Simulation Panel to observe the different protocols travelling on the network.

Note: The Capture/Forward button ‘ >| ‘ is a small arrow pointing to the right with a vertical bar next to it.

a.     Click Capture/Forward once. All of the PDUs travel to the switch.

b.     Click Capture/Forward six times and watch the PDUs from the different hosts as they travel on the network. Note that only one PDU can cross a wire in each direction at any given time.

What is this called?

It’s called a frame due to there is no router involved here and forwarding process based on mac address for every device.

A variety of PDUs appears in the event list in the Simulation Panel. What is the meaning of the different colors?

The variety pf different colors it to make it easier to distinguish between the request and response of every services and how the data transmitted through the network.

## Part 2: Examine Functionality of the TCP and UDP Protocols

### Step 1: Examine HTTP traffic as the clients communicate with the server.

a.     Click Reset Simulation.

b.     Filter the traffic that is currently displayed to only HTTP and TCP PDUs. To filter the traffic that is currently displayed:

1)    Click Edit Filters and toggle the Show All/None button.

2)    Select HTTP and TCP. Click the red “x” in the upper right-hand corner of the Edit Filters box to close it. Visible Events should now display only HTTP and TCP PDUs.

c.     Open the browser on HTTP Client and enter 192.168.1.254 in the URL field. Click Go to connect to the server over HTTP. Minimize the HTTP Client window.

d.     Click Capture/Forward until you see a PDU appear for HTTP. Note that the color of the envelope in the topology window matches the color code for the HTTP PDU in the Simulation Panel.

#### Question:

Why did it take so long for the HTTP PDU to appear?

This is a first-time connection between http client and multi-server, all devices in the network receive a request to establish a connection between the source and destination IP. All devices in the network doesn’t have the same IP for destination they will drop the packet but note the server, here server send a replay with ack. When http client receives its mean the connection is established and data can be transmitted.

e.   Click the PDU envelope to show the PDU details. Click the Outbound PDU Details tab and scroll down to the second to the last section.

What is the section labeled?

Http request

Are these communications considered to be reliable?

Yes, no chance to lose data but it’s not encrypted and can be compromised.

Record the SRC PORT, DEST PORT, SEQUENCE NUM, and ACK NUM values.

1033, 80, 1, 1

f.      Look at the value in the Flags field, which is located next to the Window field. The values to the right of the “b” represent the TCP flags that are set for this stage of the data conversation. Each of the six places corresponds to a flag. The presence of a “1” in any place indicates that the flag is set. More than one flag can be set at a time. The values for the flags are showen below.

| Flag place | 6 | 5 | 4 | 3 | 2 | 1 |
| --- | --- | --- | --- | --- | --- | --- |
| Value | URG | ACK | PSH | RST | SYN | FIN |

Which TCP flags are set in this PDU?

As I see in this filed, I got 8 digits instead of 6. If I do it right, I got 0b00011000. The ones here represent the PSH and ACK flags. PSH for sending frame and ACK for acknowledge.

g.     Close the PDU and click Capture/Forward until a PDU with a checkmark returns to the HTTP Client.

h.     Click the PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

Before it sends, source port 1033, destination port 80 and sequence number 1. When returns, source port 80, destination port 1033, sequence number 1.

i.      Click the HTTP PDU which HTTP Client has prepared to send to MultiServer. This is the beginning of the HTTP communication. Click this second PDU envelope and select Outbound PDU Details.

#### Question:

What information is now listed in the TCP section?

Same info, just ACK flag is active

How are the port and sequence numbers different from the previous two PDUs?

From previous PDU, source port 1033, destination port 80, seq number 1.

j.      Reset the simulation.

### Step 2: Examine FTP traffic as the clients communicate with the server.

a.     Open the command prompt on the FTP Client desktop. Initiate an FTP connection by entering ftp 192.168.1.254.

b.     In the Simulation Panel, change Edit Filters to display only FTP and TCP.

c.     Click Capture/Forward. Click the second PDU envelope to open it.

Click the Outbound PDU Details tab and scroll down to the TCP section.

#### Question:

Are these communications considered to be reliable?

Yes. Here, there is no sign for encrypt the data so its not secure.

d.     Record the SRC PORT, DEST PORT, SEQUENCE NUM, and ACK NUM values.

1025, 21,1,53

#### Question:

What is the value in the flag field?

00010001 which represent FIN and ACK.

e.     Close the PDU and click Capture/Forward until a PDU returns to the FTP Client with a checkmark.

f.      Click the PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

21,1026, seq 0, ack 1.

g.     Click the Outbound PDU Details tab.

#### Question:

How are the port and sequence numbers different from the previous results?

1026, 21, seq 1, ack 1

h.     Close the PDU and click Capture/Forward until a second PDU returns to the FTP Client. The PDU is a different color.

i.      Open the PDU and select Inbound PDU Details. Scroll down past the TCP section.

#### Question:

What is the message from the server?

Welcome to PT Ftp server

j.      Click Reset Simulation.

### Step 3: Examine DNS traffic as the clients communicate with the server.

a.     Repeat the steps in Part 1 to create DNS traffic.

b.     In the Simulation Panel, change Edit Filters to display only DNS and UDP.

c.     Click the PDU envelope to open it.

d.     Look at the OSI Model details for the outbound PDU.

#### Question:

What is the Layer 4 protocol?

It’s a UDP protocol.

Are these communications considered to be reliable?

In UDP the is a possibility to drop some data which in this case will not have a big effect on the data being received. Not 100% but its suitable for live streaming and VoIP calls.

e.     Open the Outbound PDU Details tab and find the UDP section of the PDU formats. Record the SRC PORT and DEST PORT values.

1027, 53

#### Question:

Why are there no sequence and acknowledgement numbers?

Due it’s a UDP protocol, there is no need for ACK from both devices and same thing with seq.

f.      Close the PDU and click Capture/Forward until a PDU with a check mark returns to the DNS Client.

g.     Click the PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

53,1027

What is the last section of the PDU called? What is the IP address for the name multiserver.pt.ptu?

DNS Answer, 192.168.1.254.

h.     Click Reset Simulation.

### Step 4: Examine email traffic as the clients communicate with the server.

a.     Repeat the steps in Part 1 to send an email to user@multiserver.pt.ptu.

b.     In the Simulation Panel, change Edit Filters to display only POP3, SMTP and TCP.

c.     Click the first PDU envelope to open it.

d.     Click the Outbound PDU Details tab and scroll down to the last section.

#### Questions:

What transport layer protocol does email traffic use?

TCP protocol.

Are these communications considered to be reliable?

Yes.

e.     Record the SRC PORT, DEST PORT, SEQUENCE NUM, and ACK NUM values. What is the flag field value?

1027, 25, 0, 0

00000010.

f.      Close the PDU and click Capture/Forward until a PDU returns to the E-Mail Client with a checkmark.

g.     Click the TCP PDU envelope and select Inbound PDU Details.

#### Question:

How are the port and sequence numbers different than before?

25, 1027, 0, 1

h.     Click the Outbound PDU Details tab.

#### Question:

How are the port and sequence numbers different from the previous two results?

1027, 25, 1, 1

i.      There is a second PDU of a different color that E-Mail Client has prepared to send to MultiServer. This is the beginning of the email communication. Click this second PDU envelope and select Outbound PDU Details.

#### Questions:

How are the port and sequence numbers different from the previous two PDUs?

1027, 25, 1, 1

What email protocol is associated with TCP port 25? What protocol is associated with TCP port 110?

Its SMTP protocol.

Its POP3 protocol.