After downloading and opening the homework1 file in Cisco Packet Tracer, I discovered that we have two distinct network segments connected via a router. The router's interface 0/0/0 is connected to Network A, while interface 0/0/1is connected to Network B.

I began by assigning static IP addresses to the devices in Network A. I configured:

- PC0 with IP address 192.168.1.3

- PC1 with IP address 192.168.1.4

- PC6 with IP address 192.168.1.5

- PC7 with IP address 192.168.1.6

- I assigned Server0 the IP address 192.168.1.2.

All devices in Network A were given the default gateway of 192.168.1.1. After completing the static IP configuration, I tested connectivity by sending a small PDU from PC0 to PC1 and from PC7 to PC6. All PDU packets were delivered successfully, confirming that the network was functioning properly.

In Network B, I followed the same steps. I began by assigning:

- PC4 the IP address 192.168.4.3

- PC5 the IP address 192.168.4.4

- Similarly, I assigned IP addresses to PC8, PC9, PC10, and PC11.

The default gateway for Network B was set to 192.168.4.1. I tested the network by sending PDUs between PC8, PC9, PC11, and PC5, as well as between PC10 and other devices. Again, all devices successfully communicated with each other, confirming proper configuration and network functionality.

After verifying connectivity between Networks A and B, I moved on to configure a DHCP server to automatically assign IP addresses instead of using static IPs. I opened the Server Configuration, navigated to the Services tab, and selected DHCP. I created two DHCP pools:

- The first pool for Network A, starting with the gateway 192.168.1.1 and a starting IP address of 192.168.1.10.

- The second pool for Network B, starting with the gateway 192.168.4.1 and a starting IP address of 192.168.4.10.

After creating the DHCP pools, I switched the devices in both networks from static IP configuration to DHCP. All devices successfully obtained IP addresses automatically from the DHCP server.

Before configuring the DNS server, I tested communication between the networks by sending PDUs from both Network A and Network B to the server. All PDUs were delivered successfully.

Now it was time to configure the DNS server. I assigned IP address 192.168.1.2 for the DNS server, I want to follow the instructions to use a single server for both networks and not adding another server. Next, I set up a simple HTTP website named "This is homework 1" using the HTTP service on the server. Then, I configured the DNS service by adding an A record for the website, associating its domain name with the server's IP address.

Once the DNS configuration was complete, I tested access to the website from devices in both Network A and Network B. The website was successfully reached from all devices, confirming that the DNS server was functioning correctly.

For VG grade….